1.1 	Instructions to Designers

This section is intended to give designers specific instructions as to how to structure the control system documents using the British Columbia Buildings Corporation standardized 	documents found in Section 1.0.

1.1.1	Technical Requirements

1.1.1.1	Table of Contents

Prepare a Table of Contents following the headings below.

1.1.1.2	15910 Scope

Prepare an overview and explanation of the CCS objectives as they relate to this project.  Note any particular features or issues which may assist the controls contractor to understand the project better.

- 15915 CCS Conditions of Contract

Attach this document unaltered.  Any revisions to this document for the particular project are to be noted in Section 15920 by the Consultant as described below.

1.1.1.4 	15920 Project Specific Alterations to BCBC - CCS Performance Specification

This is where any deviations to the BCBC standard CCS Performance Specification, Section 15940, can be made, if necessary.  This approach is used to assist the controls contractors in focusing on issues which may affect pricing of the project.

1.1.1.5	15925 Sequence of Operation

This section is to be used to describe the sequences of operation for the items to be controlled.

1.1.1.6	15930 Systems Summary and Points Lists

Summary Page - Prepare a summary of the points for each system using the Project

Summary Sheet.

System Points -  Prepare a list of points for each system and list these on a second Project

Summary Sheet.  Alternatively, the designer may use the Graphic/Logic/Points List-

One Page Format Sheets for the BCBC Pre-Approved System types.

Non DDC Points -  Prepare a list of these points using the Non DDC Points List form.

These forms are located in  Section 5.O of the manual and are also available in electronic format in Microsoft Excel Version 5.0 format.

Graphics -Where graphics are to be provided, the contractor shall create the graphics in

conformance with the BCBC Graphics Guideline Standard, see Section 7.0 in the

manual. The BCBC Pre-Approved System type one page format sheets conform to the

Graphics Guideline.

1.1.1.7	15940 BCBC - CCS Performance Specification

Attach this document unaltered.  Any revisions to this document for the particular project  are to be noted  in Section 15920 by the Consultant as described above.