3.1 	Instructions to Designers

This section is intended to give designers specific instructions as to how to structure the 	construction documents using the British Columbia Buildings Corporation standardized 	documents found in Section 3.0.

3.1.1	Instructions to Proponents - Section 3.2 - Fill in the information on the cover sheet of the 	Instructions to Proponents and the Invitation for Proposal.  The remainder of the document is 	to be issued unaltered.

3.1.2	Technical Requirements

3.1.2.1	Table of Contents - Prepare a Table of Contents following the headings below.

3.1.2.2	15910 Scope - Prepare an overview and explanation of the CCS objectives as they relate to this project.  Note any particular features or issues which may assist the controls contractor to  understand the project better.

3.1.2.3 	15920 Project Specific Alterations to BCBC - CCS Performance Specification - This is where any deviations to the BCBC standard CCS Performance Specification can be made, if necessary.  This approach is used to assist the controls contractors in focusing on issues which may affect pricing of the project.

3.1.2.4	15925 Sequence of Operation (if required) - This section is to be used if there are any strategies to be programmed by the contractor which do not  have standardized System Points Lists, Graphics & Start up Logic prepared.

3.1.2.5	15930 Systems (Points Lists, Graphics & Start up Logic)

Summary Page - Prepare a summary page of the points and graphics following the format in

the example in Section 10 (form 10_E5).

DDC Points - Choose the BCBC standardized System Points Lists, Graphic & Start up Logic

sheets from Section 5 for each mechanical or electrical system.  For more in-depth

instruction see Section 5.1.

Misc. Points - Add these points on form 5_O2 found in Section 5.O.

Non DDC Points- Add these points on form 5_O3 found in Section 5.O.

Emergency/Safety Points - Add these points on form 5_O4 found in Section 5.O.

Other Graphics - Add any other graphics required. Where standardized graphics are not in

manual, BCBC will prepare same for the project in conjunction with the designer.

3.1.2.6	15940 BCBC - CCS Performance Specification - Section 3.3 - Attach this document unaltered.